UTC Junction Description:  1003 A4 London Rd/ Cleveland Place

Outstation SCN:  X04210

Junction SCN:  J04211

IP Address:  172.16.52.49

TOPAS2500 Configuration No:  E60171 v6

Form Completed: 15 May 24

JUNCTION PHASING

H

D

C

G

E

A

# Junction Staging:

SCOOT Links:

| Link SCN | Link Description |
| --- | --- |
| N04211A | 1003 A4 London Rd/ Cleveland Place: London Rd WB Ahead |
| N04211C | 1003 A4 London Rd/ Cleveland Place: London Rd EB Rt |
| N04211D | 1003 A4 London Rd/ Cleveland Place: Cleveland Place NB |
| N04211E | 1003 A4 London Rd/ Cleveland Place: London Rd WB Lft |
| N04211G | 1003 A4 London Rd/ Cleveland Place: London Rd EB Ahead |
| N04211H | 1003 A4 London Rd/ Cleveland Place: Cleveland Place NB Stopline |

Interstage Values:				              	   Minimum Stage Lengths:

|  | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 1 | ### X | 7 | 8 | 8 | 8 |
| 2 | 5 | ### X | 8 | 8-9 | 8 |
| 3 | 11 | 11 | X | 11-12 | 11 |
| 4 | 11-13 | 11-13 | 11 | X | 11 |
| 5 | 6-8 | X | X | X | X |

|  | DD | MINS | MAXS |
| --- | --- | --- | --- |
| Stage 1 | N | 0-7 | 120 |
| Stage 2 | Y | 7 | 120 |
| Stage 3 | Y | 0-7 | 90 |
| Stage 4 | Y | 7 | 90 |
| Stage 5 | Y | 6 | 90 |

Bit Pattern:

| 1: F1 | 2: F2# | 3: F3# | 4: F4# | 5: F5# | 6: D2 | 7: D3 | 8: D4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9: D5 | 10: DX | 11: TS | 12: BF1 | 13: BF2 | 14: | 15: | 16: |

| 1: G1 | 2: G2 | 3: G3 | 4: G4 | 5: G5 | 6: DR2 | 7: DR3 | 8: DR4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9: DR5 | 10: DF | 11: CS | 12: LF1 | 13: LF2 | 14: CF | 15: RR | 16: |
| 17: SC1 | 18: | 19: | 20: | 21: SC2 | 22: | 23: | 24: |
| 25: SC3 | 26: | 27: | 28: | 29: SC4 | 30: | 31: | 32: |
| 33: SC5 | 34: | 35: | 36: | 37: SC6 | 38: | 39: | 40: |
| 41: SC7 | 42: | 43: | 44: | 45: SC8 | 46: | 47: | 48: |
| 49: SC9 | 50: | 51: | 52: | 53: SC10 | 54: | 55: | 56: |

SCOOT Plan Line:

CY120

{F1 0}1, {F2 35, F1F2 36}2, {F3 65, F2F3+D2 66}3, {F4 79, F4F5+D5 80}4, {F5 95, F1F5 96}5

H

SCOOT Stage Minimums (For Peak plans):

| SCOOT Stage | Controller Stage Change | ## Interstage + Stage Min | SCOOT Stage: Minimum Stage Length |
| --- | --- | --- | --- |
| 1 | 5 to 1 | 8 + 7 = 15 | 15 |
| 2 | 1 to 2 | 7 + 7 = 14 | 14 |
| 3 | 1 to 3 2 to 3 | 8 + 6 = 14 8 + 6 = 14 | 14 (Fixed length) |
| 4 | 2 to 4 3 to 4 | 9 + 7 = 16 12 + 7 = 19 | 16* (Fixed length) |
| 5 | 4 to 5 | 11 + 4 = 15 | 18 |

Lags:

| Controller Phase Letter | SCOOT Link Letter | Value In Spec (Time to Phase Gaining Green) | SLAG: SCOOT Value (Subtract 7) | ELAG: SCOOT Value (=Phase Delay) |
| --- | --- | --- | --- | --- |
| A | A | 5 | -2 | 0 |
| B | G | 6 | -1 | 0 |
| C | C | 7 | 0 | 4 |
| D | D/ H | 11 | +4 | 0 |
| E | E | 11 | +4 | 0 |

Notes:

Link C closes down in a fixed length stage, so can’t optimise.  If this becomes an issue, the database will need to be revised so that the SCOOT model is told it only runs in controller stage 2, and an end lag equivalent to the duration of stage 3 will need to be added.  This will enable the link to optimise during stage 2.

#### SCOOT Data

SCOOT Region SCN:  		RBL

Minimum Cycle Time:  		72 (69)

Maximum Cycle Time:		128

Use Sub Region Independence:	N

Force Site to Double Cycle?:  	N

Link & Link Stage Data:

| Link Letter | Link Type | Upstream Node | UNTL | MDSL | Junction SCN | UTC Green Stage No’s |
| --- | --- | --- | --- | --- | --- | --- |
| A | N | N04116 | I | NA | J04211 | 1 |
| C | E | NA | NA | NA | J04211 | 2,3 |
| D | N | N04235 | N | NA | J04211 | 4,5 |
| E | E | NA | NA | NA | J04211 | 1,2 |
| G | E | NA | NA | N04115O | J04211 | 1,2 |
| H | SE | NA | NA | NA | J04211 | 4,5 |

Detector Data:

| Detector SCN | O.T.U SCN | Reply Bit |
| --- | --- | --- |
| N04211A1 | X04110 | 29 |
| N04211C1 | X04210 (Sc2) IO 19/ SM 63 | 29 |
| N04211D1 | X04210 (Sc6) IO 23/ SM 59 | 41 |
| N04211E1 | X04110 | 25 |
| N04211G1 | X04210 (Sc1) IO 18/ SM 64 | 25 |
| N04211H1 | X04210 (Sc9) IO 07/ SM 53 | 45 |

Notes:

Counters added:

D04211: London Rd EB L1 @ Margarets Hill (X04210 Sc1 N04211G1) SM 64

D04212: London Rd EB L2 @ Margarets Hill (X04210 Sc2 N04211C1) SM 63

D04213: London Rd EB @ Cleveland Bridge (X04210 Sc4 N04115O1) SM 62

D04214: Cleveland Bridge SB @ London Rd (X04210 Sc5 N04235S1) SM 60

D04215: Bathwick St NB @ Cleveland Bridge (X04210 Sc6 N04211D1) SM 59

D04216: Cleveland Place NB @ London Rd (X04210 Sc9 N04211H1) SM 53

# Validation Parameters

Basic Validation Parameters:

|  | A | C | D | E | G | H |
| --- | --- | --- | --- | --- | --- | --- |
| SLAG | -2 | 0 | +4 | +4 | -1 | +4 |
| ELAG | 0 | 4 | 0 | 0 | 0 | 0 |
| JNYT | 14 | 10 | 12 | 15 | 9 | 1 |
| QCMQ | 48 | 38 | 39 | 53 | 31 | NA |
| STOC | 11* was 9 | 13 | 10 | 8 | 9 | 9 |
| CGIF | 2 | 3 | 4 | 1 | 3 | 0 |
| SMUL SSAT | NA | NA | NA | NA | NA | NA |
| FMUL FSAT | NA | NA | NA | NA | NA | NA |
| OFWT | 10 | 10 | 10 | 10 | 10 | 10 |
| DFOF BIAS | NA | NA | NA | NA | NA | NA |
| CGOF CGWT | NA | NA | NA | NA | NA | NA |
| LFLA LFAL LFEL LFMQ | NA | NA | NA | NA | NA | NA |
| FORC | No | No | No | No | No | No |
| INSP INCY INOF | NA NA NA | Off Off On | NA NA NA | Off Off On | NA NA NA | NA NA NA |

*4211a link det being clipped by 4211E link traffic

Pedestrian Priority Parameters:  NA-Out of Scope

|  | A | C | D | E | G |
| --- | --- | --- | --- | --- | --- |
| PEDS (2) |  |  |  |  |  |
| PLST |  |  |  |  |  |
| PUST |  |  |  |  |  |
| PPRI |  |  |  |  |  |
| PDCI |  |  |  |  |  |
| MPFR |  |  |  |  |  |
| MPWT |  |  |  |  |  |

Bus Priority Parameters:  NA-Out of Scope

|  | A | C | D | E | G |
| --- | --- | --- | --- | --- | --- |
| BOFF BPAN BPFL BERL |  |  |  |  |  |
| BDDD |  |  |  |  |  |
| BEXR BRER |  |  |  |  |  |
| PMAX |  |  |  |  |  |
| BJNY |  |  |  |  |  |
| BJT2 |  |  |  |  |  |
| BES0-BES6 |  |  |  |  |  |
| BRS0-BRS6 |  |  |  |  |  |

Reduced Detection Parameters:  NA-Out of Scope

|  | A | C | D | E | G |
| --- | --- | --- | --- | --- | --- |
| RDPF |  |  |  |  |  |
| RDPL RDFR RDFF JNYT |  |  |  |  |  |

Notes:

Links D & H are needed for Cleveland Bridge as left turners skew the performance of link D because they turn off after the detector (there’s no sink detector to compensate for it).  On H, they cause the opposite issue, as the vehicles that turn off cause H to see large headways between vehicles, causing it to under estimate the saturation.  Between the two links, the green split is roughly right.

Link D has N04235N set up as a congestion link for the approach.  The following values have been entered to add weight to this link during congested periods:

CHAN CLIF N04211D 3.

SCOOT Detection

Sc6: N04211H1

Sc5: N04211D1

Sc4: N04235S1

X04210

X04110

Sc3: N04115O1

Sc2: N04211C1

Sc1: N04211G1