| UTC Junction Description: | Northgate St/ New Bond St/ High St Shuttle |
| --- | --- |
| Outstation SCN & Type: | Yunex Gemini 2 – X02110 |
| Junction SCN: | J02111 |
| IP Address: | 172.16.52.48 |
| TOPAS2500 Configuration No: | E60126 v3 |
| Date Form Completed: | 29/2/24 |

JUNCTION PHASING

A

B

C

D

# Junction Staging:

SCOOT Staging: 2, 4, 5.

SCOOT Links:

| Link SCN | Link Description |
| --- | --- |
| N02111A | 1013 Northgate St/ New Bond St/ High St Bus Gate: Broad St SB |
| N02111B | 1013 Northgate St/ New Bond St/ High St Bus Gate: New Bond St EB Lft Stopline |
| N02111C | 1013 Northgate St/ New Bond St/ High St Bus Gate: New Bond St EB Rt Stopline |
| N02111D | 1013 Northgate St/ New Bond St/ High St Bus Gate: High St NB Stopline |

Interstage Values:                                       Minimum Stage Lengths:

|  | 2 | 4 | 5 |
| --- | --- | --- | --- |
| 2 | ### X | 6-26 | 8-28 |
| 4 | 6-26 | ### X | 6-26 |
| 5 | 6-26 | 6-26 | X |

|  | Demand Dependent? | MINS | MAXS |
| --- | --- | --- | --- |
| Stage 2 | Y | 10 | 120 |
| Stage 4 | Y | 10 | 120 |
| Stage 5 | Y | 10 | 120 |

Bit Pattern:

| 1: F1# | 2: F2# | 3: F3# | 4: F4# | 5: F5# | 6: F6# | 7: F7# | 8: D1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9: D2 | 10: D3 | 11: D4 | 12: D5 | 13: D6 | 14: D7 | 15: DX | 16: TS |

| 1: G1 | 2: G2 | 3: G3 | 4: G4 | 5: G5 | 6: G6 | 7: G7 | 8: DR1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9: DR2 | 10: DR3 | 11: DR4 | 12: DR5 | 13: DR6 | 14: DR7 | 15: DF | 16: CS |
| 17: EV | 18: BD1 | 19: BD2 | 20: BD3 | 21: LF1 | 22: LF2 | 23: CF | 24: RR |
| 25: SC1 | 26: | 27: | 28: | 29: SC2 | 30: | 31: | 32: |
| 33: SC2 | 34: | 35: | 36: | 37: SC4 | 38: | 39: | 40: |
| 41: VC1 | 42: VC2 | 43: VC3 | 44: VC4 | 45: | 46: | 47: | 48: |
| 49: | 50: | 51: | 52: | 53: | 54: | 55: | 56: |

Number of SCOOT detectors = 4

SCOOT Plan Lines:

| Plan No | SCOOT Plan Line |
| --- | --- |
| AM Peak | CY 112 {F2+D2 0}S1, {F4+D4 36}S2, {F5+D5 72}S3 |
| Off Peak | CY 112 {F2+D2 0}S1, {F4+D4 36}S2, {F5+D5 72}S3 |
| PM Peak | CY 112 {F2+D2 0}S1, {F4+D4 36}S2, {F5+D5 72}S3 |
| Evening | Local |

SCOOT Stage Minimums:

| SCOOT Stage | Controller Stage Change | ## Interstage + MINS + Offset | SCOOT Stage: Min Stage length |
| --- | --- | --- | --- |
| 1 | 5 to 2 | 6 to 26 + 10 = 16 to 36 | 36 |
| 2 | 2 to 4 | 6 to 26 + 10 = 16 to 36 | 36 |
| 3 | 4 to 5 | 6 to 26 + 10 = 16 to 36 | 36 |
| Total (MINC): |  |  | = 108 (112) |

Lags:

| Controller Phase Letter | SCOOT Link Letter | Value In Spec (Time to Phase Gaining Green) | SLAG: SCOOT Value (Subtract 7) | ELAG: SCOOT Value (=Phase Delay) |
| --- | --- | --- | --- | --- |
| A | A | To be measured |  | 0 |
| B | B | To be measured |  | 0 |
| C | C | To be measured |  | 0 |
| D | D | To be measured |  | 0 |

Start lags to be measured on street due to the variability.

#### SCOOT Model Data

| SCOOT Region SCN: | RBG |
| --- | --- |
| Minimum Cycle Time: | 112 |
| Maximum Cycle Time: | 128 |
| Use Sub Region Independence: | N |
| Maximum no of SCOOT Stages: | 3 |

Link & Link Stage Data:

| Link Letter | Link Type | Upstream Node | UNTL | MDSL | Junction SCN | UTC Green Stage No’s |
| --- | --- | --- | --- | --- | --- | --- |
| A | F | NA | NA | NA | J02111 | 4 |
| B | SE | NA | NA | NA | J02111 | 2 |
| C | SE | NA | NA | NA | J02111 | 2 |
| D | F | NA | NA | NA | J02111 | 5 |

Detector Data:

| Detector SCN | O.T.U SCN | SCOOT No | First Reply Bit |
| --- | --- | --- | --- |
| N02111A1 | X02110 | 1 | 25 |
| N02111B1 | X02110 | 2 | 29 |
| N02111C1 | X02110 | 3 | 33 |
| N02111D1 | X02110 | 4 | 37 |

Notes:

N02111A1 = AR5

N02111B1 = BSL

N02111C1 = CSL

N02111D1 = AR7

D02111 = N02111A1

D02112 = N02111B1

D02113 = N02111C1

D02114 = N02111D1

# Validation Parameters

Basic Validation Parameters:

|  | A | B | C | D |
| --- | --- | --- | --- | --- |
| SLAG | 5 | 5 | 5 | 5 |
| ELAG | 0 | 0 | 0 | 0 |
| JNYT | 1 | 1 | 1 | 1 |
| QCMQ | NA | NA | NA | NA |
| STOC | 8 | 8 | 8 | 8 |
| CGIF | 1 | 1 | 1 | 1 |
| SMUL SSAT | NA | NA | NA | NA |
| FMUL FSAT | NA | NA | NA | NA |
| OFWT | 10 | 10 | 10 | 10 |
| DFOF BIAS | NA | NA | NA | NA |
| CGOF CGWT | NA | NA | NA | NA |
| LFLA LFAL LFEL LFMQ | NA | NA | NA | NA |
| FORC | S | S | S | S |
| INSP INCY INOF | N N N | N N N | N N N | N N N |

Pedestrian Priority Parameters:  NA – Not in scope

|  | A | B | C | D |
| --- | --- | --- | --- | --- |
| PEDS (2) |  |  |  |  |
| PLST |  |  |  |  |
| PUST |  |  |  |  |
| PPRI |  |  |  |  |
| PDCI |  |  |  |  |
| MPFR |  |  |  |  |
| MPWT |  |  |  |  |

Bus Priority Parameters:  NA – Not in scope

|  | A | B | C | D |
| --- | --- | --- | --- | --- |
| BOFF BPAN BPFL BERL |  |  |  |  |
| BDDD |  |  |  |  |
| BEXR BRER |  |  |  |  |
| PMAX |  |  |  |  |
| BJNY |  |  |  |  |
| BJT2 |  |  |  |  |
| BES0-BES6 |  |  |  |  |
| BRS0-BRS6 |  |  |  |  |

Reduced Detection Parameters:  NA – Not in scope

|  | A | B | C | D |
| --- | --- | --- | --- | --- |
| RDPF |  |  |  |  |
| RDPL RDFR RDFF JNYT |  |  |  |  |

Notes:

SCOOT Detector Sketch:

C

B

A

X02110

D

SC2: N02111B1

SC3: N02111C1

SC4: N02111D1

SC1: N02111A1

CSI.DET.IN.1 = CSI.DET.IN.1 =

CSI.DET.IN.1 = CSI.DET.IN.1 =

CSI.DET.IN.1 = CSI.DET.IN.1 =

CSI.DET.IN.1 = CSI.DET.IN.1 =