UTC Junction Description:  Lower Bristol Rd/ Oak St Ped

Outstation SCN & Type:  ST950/ X02340

Junction SCN:  J02345

IP Address:  10.249.151.7

TOPAS2500 Configuration No:  1084 v1.12

Date Form Completed:  07/11/23

JUNCTION PHASING

O

M

A

A

B

Timing Data:

| Description* | Timing (s) |
| --- | --- |
| Traffic Stage Minimum Green | 7 |
| Traffic Stage Maximum Green | 120 |
| Ped Period 2: Leaving Amber | 3 |
| Ped Period 3: Gap Change/ Max Change (PAR) | 3 – 3 |
| Ped Period 4: Green Man (MIN) | 7 |
| Ped Period 5: Black Out (PBT) | 3 |
| Ped Period 6: On Crossing Extensions (CMX) | 10 |
| Ped Period 7: All Red (CDY) | 0 |
| Ped Period 8: All Red (CRD) | 0 |
| Ped Period 9: Starting Amber | 2 |
| SDE Extension | NA |
| RLM Extensions | NA |

|  | Min | Max | 1 | 2 |
| --- | --- | --- | --- | --- |
| Stage 1 | 7 | 120 | X | 6 |
| Stage 2 | 7 | 15 | 5 to 15 | X |

Bit Pattern:

| 1= F1 | 2= #F2 | 3= D2 | 4= TS | 5= MO | 6= | 7= | 8= MTO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9= | 10= | 11= | 12= | 13= | 14= | 15= | 16= |

| 1= G1 | 2= G2 | 3= DR2 | 4= CS | 5= CF | 6= DF | 7= LF1 | 8= LF2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9= RR | 10= MR | 11= ML | 12= MF | 13= | 14= VC1 | 15= VC2 | 16= CRB |
| 17= | 18= | 19= | 20= | 21= | 22= | 23= | 24= |
| 25= | 26= | 27= | 28= | 29= | 30= | 31= | 32= |

VC1:  D02341 = AIN1

VC2:  D02342 = AIN3

SCOOT Stage Minimums:

| SCOOT Stage | Stage Minimums |
| --- | --- |
| 1 | 28 |
| 2 | 7 |

SCOOT Plan Lines:

| Plan No | CYC | SCOOT Plan Line |
| --- | --- | --- |
| 1 | 40 | {F1 0}1, {F2 33, F1F2 34}2 |

#### SCOOT Data

SCOOT Region SCN:  		RLE (Lower Bristol Rd East)

Use Sub Region Independence:	N

Minimum Cycle Time:  		36

Maximum Cycle Time:		120

No of Stages:  			2

Force Site to Double Cycle?:  	N

Link & Link Stage Data:

| Link Letter | Link Type | Upstream Node | MDSL | UNTS | DNTS | Junction SCN | UTC Green Stage No’s | Gains Green in Stage | Loses Green in Stage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M | E | - | NA | NA | NA | J02345 | 1 | 1 | 2 |
| O | N | - | NA | NA | NA | J02345 | 1 | 1 | 2 |

Detector Data:

| Detector SCN | O.T.U SCN | Card/ Bit | Reply Bit |
| --- | --- | --- | --- |
| N02345M1 | X02340 (AIN1) | Bit 1 | 17 |
| N02345O1 | X02340 (AIN3) | Bit 3 | 21 |

Notes:

# Validation Parameters

Basic Validation Parameters:

|  | M | O |
| --- | --- | --- |
| SLAG |  |  |
| ELAG |  |  |
| JNYT |  |  |
| QCMQ |  |  |
| STOC |  |  |
| CGIF |  |  |
| SMUL SSAT |  |  |
| FMUL FSAT |  |  |
| OFWT |  |  |
| DFOF BIAS |  |  |
| CGOF CGWT |  |  |
| LFLA LFAL LFEL LFMQ |  |  |
| FORC |  |  |
| INSP INCY INOF |  |  |

Pedestrian Priority Parameters:  NA-Out of Scope

|  | M | O |
| --- | --- | --- |
| PEDS (2) |  |  |
| PLST |  |  |
| PUST |  |  |
| PPRI |  |  |
| PDCI |  |  |
| MPFR |  |  |
| MPWT |  |  |

Bus Priority Parameters:  NA-Out of Scope

|  | M | O |
| --- | --- | --- |
| BOFF BPAN BPFL BERL |  |  |
| BDDD |  |  |
| BEXR BRER |  |  |
| PMAX |  |  |
| BJNY |  |  |
| BJT2 |  |  |
| BES0-BES6 |  |  |
| BRS0-BRS6 |  |  |

Reduced Detection Parameters:  NA-Out of Scope

|  | M | O |
| --- | --- | --- |
| RDPF |  |  |
| RDPL RDFR RDFF JNYT |  |  |

Notes: