UTC Junction Description:  5040 A37 Wells Rd/ A39 White Post (Hallatrow)

Outstation SCN & Type:  X88820 - Stratos

Junction SCN:  J88821

IP Address:  172.16.53.40

TOPAS2500 Configuration No:  TCT06 v1

Date Form Completed:  15/08/2022

JUNCTION PHASING

E

D

C

B

A

# Junction Staging:

SCOOT Links:

| Link SCN | Link Description |
| --- | --- |
| N88821A | 5040 A37 Wells Rd/ A39 White Cross (Hallatrow): A37 SB |
| N88821B | 5040 A37 Wells Rd/ A39 White Cross (Hallatrow): A37 NB Ahead |
| N88821C | 5040 A37 Wells Rd/ A39 White Cross (Hallatrow): A37 NB Rt |
| N88821D | 5040 A37 Wells Rd/ A39 White Cross (Hallatrow): A39 WB |
| N88821E | 5040 A37 Wells Rd/ A39 White Cross (Hallatrow): Car Wash Exit |

Interstage Values:					Minimum Stage Lengths:

|  | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| 1 | ### X | 6 | 7 | 7 |
| 2 | 7 | ### X | 7 | 6 |
| 3 | 6 | 6 | X | 7 |
| 4 | 9 | 9 | 9 | X |

|  | DD | MINS | MAXS |
| --- | --- | --- | --- |
| Stage 1 | N | 7 | 120 |
| Stage 2 | Y | 7 | 60 |
| Stage 3 | Y | 7 | 90 |
| Stage 4 | Y | 7 | 30 |

Bit Pattern No:

| 0= F1 | 1= F2 | 2= F3 | 3= F4 | 4= F5 | 5= D2 | 6= D3 | 7= D4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8= DX | 9= TS | 10=MO | 11=BF1 | 12=BF2 | 13=BF3 | 14=BF4 | 15= |

| 0= G1 | 1= G2 | 2= G3 | 3= G4 | 4= G5 | 5= DR2 | 6= DR3 | 7= DR4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8= CS | 9= CF | 10= DF | 11= LF1 | 12= LF2 | 13= RR | 14= MR | 15= ML |
| 16= MF | 17= | 18= | 19= | 20= | 21= | 22= | 23= |
| 24= | 25= | 26= | 27= | 28= | 29= | 30= | 31= |

SCOOT Plan Lines:

| Plan No | SCOOT Plan Line |
| --- | --- |
| 1 | {F1 0}1, {F2 0, F1/2 1}2, {F3 0+D3}3, {F4 0, F1/4 1}4 |

SCOOT Stage Minimums:

| SCOOT Stage | Controller Stage Change | ## Interstage + Stage Min | SCOOT Stage: Minimum Stage Length |
| --- | --- | --- | --- |
| 1 | 3 – 1 4 – 1 | 6 + 7 = 13 9 + 7 = 16 | 16 |
| 2 | 1 – 2 | 6 + 7 = 13 | 13 |
| 3 | 1 – 3 2 – 3 | 7 + 7 = 14 7 + 7 = 14 | 14 |
| 4 | 3 – 4 | 7 + 7 = 14 | 14 |

Lags:

| Controller Phase Letter | SCOOT Link Letter | Value In Spec (Time to Phase Gaining Green) | SLAG: SCOOT Value (Subtract 7) | ELAG: SCOOT Value (=Phase Delay) |
| --- | --- | --- | --- | --- |
| A | A | 9 | +2 | 0 |
| B | B | 9 | +2 | 0 |
| C | C | 6 | -1 | 0 |
| D | D | 7 | 0 | 0 |
| E | E | 7 | 0 | 0 |

#### SCOOT Data

SCOOT Region SCN:  		RWR

Minimum Cycle Time:  		60

Maximum Cycle Time:		120

Use Sub Region Independence:	N

Initial Region Cycle Time (IRCT):  	120

Force Site to Double Cycle?:  	N

SCOOT Stage Timing Data:

| SCOOT Stage | Minimum Stage Length (MINS) | Maximum Stage Length (MAXS) | Stage Change Point: |
| --- | --- | --- | --- |
| 1 | 16 | 120 | 0 |
| 2 | 13 | 60 | 50 |
| 3 | 14 | 90 | 65 |
| 4 | 14 | 30 | 105 |

Link & Link Stage Data:

| Link Letter | Link Type | Upstream Node | MDSL | UNTS | DNTS | Junction SCN | UTC Green Stage No’s | Gains Green in Stage | Loses Green in Stage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | E | NA | NA | NA | 1 | J88821 | 1 | 1 | 2 |
| B | E | NA | NA | NA | 1 | J88821 | 1 | 1 | 2 |
| C | E | NA | NA | NA | 2 | J88821 | 2 | 2 | 3 |
| D | E | NA | NA | NA | 3 | J88821 | 3 | 3 | 4 |
| E | E | NA | NA | NA | 4 | J88821 | 4 | 4 | 1 |

Detector Data:

| Detector SCN | O.T.U SCN | Reply Word | Reply Mask |
| --- | --- | --- | --- |
| N88821A1 | X88820 | 2 | 0 |
| N88821B1 | X88820 | 2 | 1 |
| N88821C1 | X88820 | 2 | 2 |
| N88821D1 | X88820 | 2 | 3 |

Notes:

# Validation Parameters

Basic Validation Parameters:

|  | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- |
| SLAG | +2 | +2 | -1 | 0 | 0 |
| ELAG | 0 | 0 | 0 | 0 | 0 |
| JNYT | 8 | 8 | 5 | 8 | 5 |
| QCMQ | 26 | 26 | 17 | 26 | 16 |
| STOC | 8 | 8 | 8 | 8 | 8 |
| CGIF | 3 | 3 | 2 | 1 | 1 |
| SMUL SSAT | NA | NA | NA | NA | NA |
| FMUL FSAT | NA | NA | NA | NA | NA |
| OFWT | 10 | 10 | 10 | 10 | 10 |
| DFOF BIAS | NA | NA | NA | NA | NA |
| CGOF CGWT | NA | NA | NA | NA | NA |
| LFLA LFAL LFEL LFMQ | NA | NA | NA | NA | NA |
| FORC | NA | NA | NA | NA | NA |
| INSP INCY INOF | NA | NA | NA | NA | NA |

Pedestrian Priority Parameters:  NA-Out of Scope

|  | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- |
| PEDS (2) |  |  |  |  |  |
| PLST |  |  |  |  |  |
| PUST |  |  |  |  |  |
| PPRI |  |  |  |  |  |
| PDCI |  |  |  |  |  |
| MPFR |  |  |  |  |  |
| MPWT |  |  |  |  |  |

Bus Priority Parameters:

|  | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- |
| BOFF BPAN BPFL BERL |  |  |  |  |  |
| BDDD |  |  |  |  |  |
| BEXR BRER |  |  |  |  |  |
| PMAX |  |  |  |  |  |
| BJNY |  |  |  |  |  |
| BJT2 |  |  |  |  |  |
| BES0-BES6 |  |  |  |  |  |
| BRS0-BRS6 |  |  |  |  |  |

Reduced Detection Parameters:  NA-Out of Scope

|  | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- |
| RDPF |  |  |  |  |  |
| RDPL RDFR RDFF JNYT |  |  |  |  |  |

Notes:

SCOOT Model created for monitoring on MOVA junction only.

SCOOT Detection:

D88824

D88823

D88822

D88821

SC4: N88821D1

(DIN9)

SC3: N88821C1

(CX7)

C

SC2: N88821B1

(BIN4)

SC1: N88821A1

(AIN1)

A

E

B

D

X88820