| UTC Junction Description: | Silver St/ Lynwood Close Ped |
| --- | --- |
| Outstation SCN & Type: | Swarco PTC-1 – X97710 |
| Junction SCN & Type: | J97711 |
| IP Address: | 172.16.53.50 |
| TOPAS2500 Configuration No: | 5062_Norton_Hill_School_cfg_4041.cpf Dated 11/19/2020 |
| Form Completed on Date: | Jul 24 |

JUNCTION PHASING

B

B

C

A

TIMING DATA:

| A Description* | Timing (s) |
| --- | --- |
| Traffic Stage Minimum Green | 7 |
| Traffic Stage Maximum Green | 120 |
| Ped Period 2: Leaving Amber | 3 |
| Ped Period 3: Gap Change/ Max Change (PAR) | 1-3 |
| Ped Period 4: Green Man (MIN) | 5 |
| Ped Period 5: Black Out (PBT) | 3 |
| Ped Period 6: On Crossing Extensions (CMX) | 10 |
| Ped Period 7: All Red (CDY) | 0 |
| Ped Period 8: All Red (CRD) | 0 |
| Ped Period 9: Starting Amber | 2 |
| SDE Extension | NA |
| RLM Extensions | 2 |
| Ped Stage: Total Lower/ Upper Not-GX Time | = 14 to 28 |

* The timing periods will vary depending on the type of pedestrian crossing.

Bit Pattern:

| 1= F1 | 2= F2# | 3= D2 | 4= DX | 5= TS | 6= MO | 7= | 8= |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9= | 10= | 11= | 12= | 13= | 14= | 15= | 16= |

| 1= G1 | 2= G2 | 3= DR2 | 4= DF | 5= CS | 6= RR | 7= CF | 8= LF1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9= LF2 | 10= LO | 11= MR | 12= MF | 13= ML | 14= VC1 | 15= VC2 | 16= |
| 17= VS1 | 18= | 19= | 20= | 21= VS2 | 22= | 23= | 24= |
| 25= | 26= | 27= | 28= | 29= | 30= | 31= | 32= |

SCOOT Plan Line:

CY032 {F1 0}S1, {F2 18, F1F2 19}S2

SCOOT Stage Minimums:

| SCOOT Stage | Controller Stage Change | SCOOT Stage: Min Stage Length | SCOOT Stage: Max Stage Length |
| --- | --- | --- | --- |
| V | 2 to 1= (MinG + Upper Not GX) – Lower Not GX | 21 | 120 |
| P | 1 to 2= Lower Not GX | 14 | 14 |

#### SCOOT Data

| SCOOT Region SCN: | RNS |
| --- | --- |
| Minimum Cycle Time: | 32 |
| Maximum Cycle Time: | 120 |
| Use Sub Region Independence: | N |
| Maximum no of SCOOT Stages: | 2 |

Link & Link Stage Data:

| Link Letter | Link Type | Upstream Node | UNTL | MDSL | Junction SCN | UTC Green Stage No’s |
| --- | --- | --- | --- | --- | --- | --- |
| A | E | NA | NA | NA | J97711 | 1 |
| B | E | NA | NA | NA | J97711 | 1 |

Detector Data:

| Detector SCN | O.T.U SCN | First Reply Bit |
| --- | --- | --- |
| N97711A1 | X97710 | 17 |
| N97711B1 | X97710 | 21 |

Notes:

# Validation Parameters

Basic Validation Parameters:

|  | A | B |
| --- | --- | --- |
| SLAG | 0 | 0 |
| ELAG | 0 | 0 |
| JNYT | 8 | 8 |
| QCMQ | 25 | 25 |
| STOC | 8 | 8 |
| CGIF | 1 | 1 |
| SMUL SSAT | NA | NA |
| FMUL FSAT | NA | NA |
| OFWT | 10 | 10 |
| DFOF BIAS | NA | NA |
| CGOF CGWT | NA | NA |
| LFLA LFAL LFEL LFMQ | N | N |
| FORC | R | R |
| INSP INCY INOF | N N N | N N N |

Pedestrian Priority Parameters:  NA – Is configured as a junction

|  | A | B |
| --- | --- | --- |
| PEDS (2) |  |  |
| PLST |  |  |
| PUST |  |  |
| PPRI |  |  |
| PDCI |  |  |
| MPFR |  |  |
| MPWT |  |  |

Bus Priority Parameters:  NA – Not in Scope

|  | A | B |
| --- | --- | --- |
| BOFF BPAN BPFL BERL |  |  |
| BDDD |  |  |
| BEXR BRER |  |  |
| PMAX |  |  |
| BJNY |  |  |
| BJT2 |  |  |
| BES0-BES6 |  |  |
| BRS0-BRS6 |  |  |

Reduced Detection Parameters:  NA – Not in Scope

|  | A | B |
| --- | --- | --- |
| RDPF |  |  |
| RDPL RDFR RDFF JNYT |  |  |

Notes: